Verilog ALU — Week 1 Reference

A complete refresher on everything built and learned in Week 1

Project: 4-bit ALU  |  Language: Verilog  |  Tool: EDA Playground (Icarus Verilog 12.0)

# 1. What We Built

A 4-bit ALU (Arithmetic Logic Unit) — the core compute block inside every CPU. It takes two 4-bit numbers and an operation selector, and outputs a 4-bit result plus a zero flag.

Operations supported:

- 000 → ADD  (A + B)

- 001 → SUB  (A - B)

- 010 → AND  (A & B)

- 011 → OR   (A | B)

- 100 → XOR  (A ^ B)

- 101 → LEFT SHIFT  (A << 1)

- 110 → RIGHT SHIFT (A >> 1)

# 2. The ALU Module (design.sv)

module alu_4bit ( input [3:0] A, // 4-bit operand A input [3:0] B, // 4-bit operand B input [2:0] op, // 3-bit operation selector output reg [3:0] result, // 4-bit result output reg zero // 1 if result == 0 ); always @(*) begin case (op) 3'b000: result = A + B; 3'b001: result = A - B; 3'b010: result = A & B; 3'b011: result = A ¦ B; 3'b100: result = A ^ B; 3'b101: result = A << 1; 3'b110: result = A >> 1; default: result = 4'b0000; endcase zero = (result == 4'b0000) ? 1'b1 : 1'b0; end endmodule

# 3. Keyword Quick Reference

| Keyword | Meaning |
| --- | --- |
| module / endmodule | Defines a hardware block — like a chip with named pins |
| input | Signal coming INTO the module (driven from outside) |
| output | Signal going OUT of the module (observed from outside) |
| reg | Holds/stores a value — used for outputs you assign inside always blocks |
| wire | Passes a signal through — used for outputs you just observe |
| [3:0] | 4-bit wide signal (bits 3 down to 0). [2:0] = 3-bit, [1:0] = 2-bit |
| always @(*) | Run this block whenever ANY input signal changes (combinational) |
| always @(posedge clk) | Run this block only on rising clock edge (sequential) |
| case / endcase | Switch statement — selects logic based on op value |
| default | Fallback case if no other case matches |
| = (blocking) | Instant assignment — used in combinational always blocks |
| <= (non-blocking) | Clock-edge assignment — used in sequential always blocks |
| #10 | Wait 10 time units (testbench only — not real hardware) |
| $display(...) | Print to simulation log — like printf in C |
| $dumpfile / $dumpvars | Record all signals to a .vcd waveform file |
| $finish | End the simulation |
| 4'b0011 | 4-bit binary literal = decimal 3. Format: width'base value |
| posedge / negedge | Rising edge / falling edge of a clock signal |

# 4. Combinational vs Sequential

## Combinational Logic

Output changes instantly whenever input changes. No clock. No memory.

always @(*) begin // react to any signal change result = A + B; // = blocking, instant end

## Sequential Logic

Output only updates on the rising edge of the clock. Has memory. Holds state.

always @(posedge clk) begin // react only to clock rising edge result <= A + B; // <= non-blocking, waits for clock end

Real-world analogy: Combinational = light switch (instant). Sequential = camera shutter (captures only at the click).

# 5. The Testbench (testbench.sv)

The testbench is separate hardware that connects to your module and drives inputs to verify outputs. Think: module = chip on PCB, testbench = signal generator + oscilloscope on your lab bench.

## Key Rules

- Inputs in testbench → declared as reg (you drive/control them)

- Outputs in testbench → declared as wire (you just observe them)

- .A(A) syntax connects testbench signal to module port

- #10 gives the module time to compute before you read the output

- Always predict expected output BEFORE running — that's the verification mindset

## Testbench Structure

`timescale 1ns / 1ps module alu_4bit_tb; reg [3:0] A, B; reg [2:0] op; wire [3:0] result; wire zero; alu_4bit uut (.A(A), .B(B), .op(op), .result(result), .zero(zero)); initial begin $dumpfile("alu.vcd"); $dumpvars(0, alu_4bit_tb); A = 4'b0011; B = 4'b0101; op = 3'b000; #10; $display("ADD: %b + %b = %b ¦ zero=%b", A, B, result, zero); // ... more tests ... $finish; end endmodule

# 6. Expected Test Outputs

ADD: 0011 + 0101 = 1000 ¦ zero=0 // 3+5=8 SUB: 0111 - 0011 = 0100 ¦ zero=0 // 7-3=4 AND: 1100 & 1010 = 1000 ¦ zero=0 OR: 1100 ¦ 0011 = 1111 ¦ zero=0 XOR: 1100 ^ 1010 = 0110 ¦ zero=0 LEFT SHIFT: 0011 << 1 = 0110 ¦ zero=0 RIGHT SHIFT: 1100 >> 1 = 0110 ¦ zero=0 ZERO FLAG TEST: 0000 + 0000 = 0000 ¦ zero=1

# 7. Common Mistakes to Avoid

- Copy-paste bug: changing op values when duplicating test cases (most dangerous — test passes but tests wrong thing)

- Using 8-bit result when ALU is 4-bit — overflow just gets dropped in hardware

- Forgetting semicolons after assignments in Verilog

- Using = instead of <= in sequential (clocked) blocks

- Not updating bit-width prefix when expanding op (2'b → 3'b)

# 8. Week 2 Preview — Sequential Logic

Week 2 introduces a clock signal — the heartbeat of all real digital hardware. You will build:

- A D flip-flop — the basic memory cell

- A 4-bit register — stores the ALU result until next clock edge

- Wire the register to your ALU — making it behave like a real CPU component

The key new concept: your ALU result will no longer update instantly. It will hold its value until the next clock tick, then latch the new result. That is sequential logic.

Week 1 complete  ·  Verilog ALU  ·  EE Portfolio